ISO/DIS 15926-100(en)

First edition

Date: 2025-05-30

Industrial automation systems and integration — Integration of life-cycle data for process plants including oil and gas production facilities —

Part 100:
Vocabulary

© ISO 2025

All rights reserved. Unless otherwise specified, or required in the context of its implementation, no part of this publication may be reproduced or utilized otherwise in any form or by any means, electronic or mechanical, including photocopying, or posting on the internet or an intranet, without prior written permission. Permission can be requested from either ISO at the address below or ISO’s member body in the country of the requester.

ISO copyright office

CP 401 • Ch. de Blandonnet 8

CH-1214 Vernier, Geneva

Phone: + 41 22 749 01 11

E-mail: copyright@iso.org

Website: www.iso.org

Published in Switzerland

Contents

Foreword	iv

Introduction	v

1	Scope	1

2	Normative references	1

3	Terms and definitions	1

3.1	Terms relating to basic concept	1

3.2	Terms relating to individual and class	4

3.3	Terms relating to artefact, product and material object	8

3.4	Terms relating to part and components	8

3.5	Terms relating to aggregations of parts	8

3.6	Terms relating to activity and participation	8

3.7	Terms relating to person, organization, position and role in organization	9

3.8	Terms relating to requirement, design and specification	10

3.9	Terms relating to method and plan	11

3.10	Terms relating to process and service	14

3.11	Terms relating to geometric and topological representation	14

3.12	Terms relating to physical quantity and physical property	15

3.13	Terms relating to implementation	16

Annex A (informative)  Development Approach	25

Bibliography	30

Foreword

ISO (the International Organization for Standardization) is a worldwide federation of national standards bodies (ISO member bodies). The work of preparing International Standards is normally carried out through ISO technical committees. Each member body interested in a subject for which a technical committee has been established has the right to be represented on that committee. International organizations, governmental and non-governmental, in liaison with ISO, also take part in the work. ISO collaborates closely with the International Electrotechnical Commission (IEC) on all matters of electrotechnical standardization.

The procedures used to develop this document and those intended for its further maintenance are described in the ISO/IEC Directives, Part 1. In particular, the different approval criteria needed for the different types of ISO documents should be noted. This document was drafted in accordance with the editorial rules of the ISO/IEC Directives, Part 2 (see www.iso.org/directives).

ISO draws attention to the possibility that the implementation of this document may involve the use of (a) patent(s). ISO takes no position concerning the evidence, validity or applicability of any claimed patent rights in respect thereof. As of the date of publication of this document, ISO had not received notice of (a) patent(s) which may be required to implement this document. However, implementers are cautioned that this may not represent the latest information, which may be obtained from the patent database available at www.iso.org/patents. ISO shall not be held responsible for identifying any or all such patent rights.

Any trade name used in this document is information given for the convenience of users and does not constitute an endorsement.

For an explanation of the voluntary nature of standards, the meaning of ISO specific terms and expressions related to conformity assessment, as well as information about ISO's adherence to the World Trade Organization (WTO) principles in the Technical Barriers to Trade (TBT), see www.iso.org/iso/foreword.html.

This document was prepared by Technical Committee ISO/TC 184, Automation systems and integration, Subcommittee SC 4, Industrial data.

A list of all parts in the ISO 15926 series can be found on the ISO website.

Any feedback or questions on this document should be directed to the user’s national standards body. A complete listing of these bodies can be found at www.iso.org/members.html.

Introduction

ISO 15926 is an International Standard for the representation of process plant life-cycle information. This representation is specified by a generic, conceptual data model that is suitable as the basis for implementation in a shared database or data warehouse.

The data model is designed to be used in conjunction with reference data: standard instances that represent data common to a substantial number of experts from the process plant engineering supply chain.

The support for a specific life-cycle activity depends on the use of an appropriate selection of reference data in conjunction with an appropriate data model derived from the ISO 15926 data model ontology.

ISO 15926 is organized as a series of parts:

ISO 15926-1: Overview and fundamental principles, published in 2004

ISO 15926-2: Data model, published in 2003

ISO/TS 15926-3: Reference data for geometry and topology, published in 2009

ISO/TS 15926-4: Core reference data, published in 2024

ISO 15926-5 has been replaced by an annex to ISO TC184/SC4: Procedure for development and maintenance of reference data in database format.

ISO 15926-6: Methodology for the development and validation of reference data.

ISO/TS 15926-7: Implementation methods for the integration of distributed systems: Template methodology, published in 2011

ISO/TS 15926-8: Implementation methods for the integration of distributed systems: Web Ontology Language (OWL) implementation, published in 2011

ISO/TS 15926-9: Implementation methods for the integration of distributed systems - Facade implementation, not yet in ballot

ISO 15926-10: Conformance testing, published in 2019

ISO/TS 15926-11: Simplified industrial usage of reference data based on RDFS methodology, published in 2013

ISO/TS 15926-12: Life-cycle integration ontology represented in OWL, published in 2018

ISO 15926-13: Integrated asset planning life-cycle, published in 2018

ISO/TR 15926-14: Data model adapted for OWL 2 Direct Semantics, cancelled in 2023

ISO 15926-100: Vocabulary. Under preparation

This document lists terms used in the ISO 15926 parts. The terms are grouped based on ISO/TS 23164:2025[1]. All terms of published ISO 15926 parts are indexed in the MS Excel and JSON file obtained by dereferencing the URI:

https://standards.iso.org/iso/15926/-100/ed1/en/index_of_ISO_15926_terms

The approach used to develop the content of this document is described in Annex A.

The target audiences for this document are implementers of the ISO 15926 series and relevant standard developers.

The terms in this document will be updated in accordance with any new or revised parts that are developed, and revised editions will be released on a regular basis.

Industrial automation systems and integration — Integration of life-cycle data for process plants including oil and gas production facilities —

Part 100:
Vocabulary

# Scope

This document defines terms relating to integration of life-cycle data for process plants. These terms are used by the parts in the ISO 15926 series.

The following are outside the scope of this document:

the reference data items that are contained in the reference data library, such as ISO/TS 15926-4:2024[2];

the entities used in the data model, such as .ISO 15926-2:2003[3].

# Normative references

There are no normative references in this document.

# Terms and definitions

ISO and IEC maintain terminological databases for use in standardization at the following addresses:

IEC Electropedia: available at http://www.electropedia.org/

ISO Online browsing platform: available at http://www.iso.org/obp

## Terms relating to basic concept

4-Dimensionalism

data modelling approach that an object's persistence through time is like its extension (3.1.9) through space

asset

item, thing  (3.1.26) or entity (3.1.8) that has potential or actual value (3.13.63) to an organization

[SOURCE: ISO 55000:2024[4], 3.1.1, modified — Note 1 to 2 to entry have been deleted.]

conceptual data model

data model in the three schema architecture defined by ISO TR 9007:1987[5]in which the structure of data (3.1.4) is represented in a form independent of any physical storage or external presentation format

data

representation of information (3.1.11) in a formal manner suitable for communication, interpretation, or processing by human beings or computers

[SOURCE: ISO 10303-2:2024[6], 3.1.207]

document

thing (3.1.26) serving as a representation of information (3.1.11) by means of symbolic marks

Note 1 to entry: The word “document” is used in a wider sense. Next to the information (3.1.11) content of customary (paper) documents (3.1.5) (not a paper document (3.1.5) itself), such as equipment data sheets or purchase orders, it can also be used for other sets of data (3.1.4), like the transaction data (3.1.4) that are input to an engineering (3.8.3) program or data (3.1.4) sets that are exchanged between systems of business partners.

engineering data

data (3.1.4) that represents the design and or engineering (3.8.3) of a system or a system element (3.5.2)

Note 1 to entry: The scope can be limited to a specific discipline (electrical, mechanical, civil), however after integrating all engineering data (3.1.6) obtained from engineering (3.8.3) tools, the result should represent the integrated design in a consistent way, which implies appropriate quality and harmonization of the data (3.1.4), obtained from the various tools.

entity

<data model> class (3.2.1) of information (3.1.11) defined by common properties (3.12.3)

Note 1 to entry: In OWL[7], which is used by some ISO 15926 parts for implementation, an entity is a storage unit that an XML (Extensible Markup Language) document might consist of.

[SOURCE: ISO 10303-2:2024[6], 3.2.9, Modified - Note 1 of entry has been added.]

entity

<linguistics> something that exists separately from other things and has its own identity

Note 1 to entry: This definition is from ISO/TS 15926-11:2023[8].

extension

totality of objects to which a concept corresponds

Note 1 to entry: ISO/TS 15926-4:2024[2]is an extension (3.1.9) of ISO 15926-2:2003[3], wherein all reference data items (3.1.21) are classified as members (3.2.10) of ISO 15926-2:2003[3] entities.

[SOURCE: ISO 22745-2:2010[9], 4.3, modified — Note 1 to entry has been added.]

facility

permanent, semi-permanent, or temporary commercial or industrial property (3.12.3) built, established, or installed for the performance of one or more specific activities (3.6.1) or functions

EXAMPLE	A building, plant, or structure.

information

facts, concepts, or instructions

[SOURCE: ISO 10303-2:2024[6], 3.1.227]

interoperability

<information technology> ability of different types of computers, networks, operating systems, and applications to work together effectively, without prior communication, in order to exchange information (3.1.11) in a useful and meaningful manner

interoperability

<system engineering> ability of effective interaction between systems based on the exchange of information (3.1.11)

Note 1 to entry: Systems can be computerized systems or enterprises.

Note 2 to entry: This definition is from ISO/TS 15926-11:2024.

life-cycle information

information (3.1.11) about an individual (3.2.6), collected at any point in time during the life-cycle of that individual (3.2.6)

metadata

data (3.1.4) that describes and defines other data (3.1.4)

[SOURCE: ISO/IEC 11179-1:2023[10], 3.2.26]

model

<linguistics> simplified description, especially a mathematical one, of a system or process, to assist calculations and predictions

ontology

<industrial data> formal statement (3.13.49) of an understanding of the world

Note 1 to entry: An ontology (3.1.17) can be represented in any language. It need not be represented in a language specifically designed for ontologies (3.1.17), such as OWL. An ontology (3.1.17) can have different representations.

Note 2 to entry: An ontology (3.1.17) does not specify what data (3.1.4) need to be recorded about the world.

Note 3 to entry: The ontology (3.1.17) is principally concerned with the world outside a computer system.

ontology

<ISO 15926 implementation methods with template methodology and OWL> formal representation of a set of concepts within a domain and the relationships between those concepts

Note 1 to entry: Ontologies are usually used to reason about the properties of that domain, and can be used to define the domain.

Note 2 to entry: Ontologies are usually expressed in a logic-based language, but this is not a requirement, neither is the need for reasoning capability. In addition to relationships, classes, properties, instances and axioms can be used.

Note 3 to entry: This definition is from ISO/TS 15926-8:2011[11].

process plant life-cycle data

data (3.1.4) that represents, in computer-processable form, information (3.1.11) about one or more process plants in or throughout any phase or phases of a process plant's life-cycle, including design, engineering (3.8.3), construction, operation, maintenance, decommissioning and demolition

reference data

process plant life-cycle data (3.1.19) that represent information (3.1.11) about classes (3.2.1) or individuals (3.2.6) things which are common to many facilities or of interest to many users

reference data item

RDI

thing (3.1.26) that is defined within a reference data library (3.1.22)

reference data library

RDL

managed collection of reference data items (3.1.21)

relationship

connection among model (3.1.16) elements

Note 1 to entry: ISO 15926-2:2003[3] entity “relationship” is something that one thing (3.1.26) has to do with another. ISO 15926-2:2003[3] uses the spatio-temporal paradigm. ISO 15926-2:2003[3] entity “relationship” does not exist in space-time.

[SOURCE: ISO/IEC 11179-3:2023[12], 3.1.4, modified — Note 1 to entry has been rewritten.]

taxonomy

collection of controlled vocabulary terms organized in a hierarchical structure, where each term is in one or more parent/child (broader/narrower) relationship to other terms in the taxonomy (3.1.24)

term

designation that represents a general concept by linguistic means

EXAMPLE	“laser printer”, “planet”, “pacemaker”, “chemical compound”, “¾ time”, “Influenza A virus”, “oil painting”.

[SOURCE: ISO 1087:2019[13], 3.4.2, modified — Note 1 to entry has been deleted.]

thing

anything that is, or can be thought about, or perceived

Note 1 to entry:  A thing (3.1.26) can be a material or non-material object, idea or action.

## Terms relating to individual and class

class

kind

category or division of things (3.1.26) based on one or more criteria for inclusion and exclusion

Note 1 to entry: A class (3.2.1) need not have any members (3.2.10).

Note 2 to entry: Because a spatio-temporal paradigm is used to define individuals in this document, all classes are non-well-founded sets. These are explained in ISO 15926-2:2003[3], D.2.4.

classification

type of relationship (3.1.23) that indicates that the classified thing (3.1.26) is a member (3.2.10) of the classifier class (3.2.1)

Note 1 to entry: Classification (3.2.2) is not transitive.

commodity product class

manufactured product class (3.2.9) that has sufficient characterization to indicate suitability for a defined use, and that is an open agreed standard

EXAMPLE	The type of lightbulb known as 60 W 230 V E27 is a commodity product class (3.2.3).

Note 1 to entry: Often a commodity product class (3.2.3) is defined by several other standards. A piping component typically conforms to a “shape standard” and a “material standard”.

Note 2 to entry: Different manufactured product classes (3.2.9), which are specializations of the same commodity product class (3.2.3), are interchangeable for the usage defined by the commodity product class (3.2.3).

core class

class (3.2.1) that is a commonly used subdivision corresponding to terms (3.1.25) used in common language

EXAMPLE	Pipe, floor, pump, and light bulb are all core classes (3.2.4).

de facto class

class (3.2.1) corresponding to common natures that are widely recognized but not formally agreed or defined

EXAMPLE	USB port and HB pencil are de facto classes (3.2.5).

Note 1 to entry: De facto classes (3.2.5) can be subsequently formalized by international, national, or industry agreement.

Note 2 to entry: A manufacturer can choose to make a product of similar specification to that of another manufacturer in order to compete for the market share by choosing to conform to some characteristics of the other product.

individual

particular

thing (3.1.26) that exists in space and time

EXAMPLE	A pump with serial number ABC123, Battersea Power Station, Sir Joseph Whitworth, and the Starship "Enterprise" are examples of individuals (3.2.6).

Note 1 to entry: In this context, existence could be within the world we live in, or some “possible” world that can be imagined. This therefore includes actual, hypothetical, planned, expected, or required individuals (3.2.6)

Note 2 to entry: particular (3.2.6) is added as an admitted term (3.1.25) to align with ISO/TS 23164:2025[1].

instance

<integration of life-cycle data for process plants> data (3.1.4) that represents, in computer-processable form, some real-world thing (3.1.26)

Note 1 to entry: In ISO 15926-2:2003[3], member (3.2.10) and instance (3.2.7) both mean member (3.2.10) of a class (3.2.1), instance (3.2.7) is used when the class (3.2.1) is an entity type in the conceptual data model (3.1.3), whilst member (3.2.10) of means there is an instance (3.2.7) of the classification (3.2.2) entity type between instance (3.2.7) of the conceptual data model (3.1.3).

Note 2 to entry: This is different from the ISO 10303-2:2024[6]definition due to its use for reference (3.13.33) and life-cycle data (3.1.4).

instance

<product data representation and exchange> named value (3.13.63)

Note 1 to entry: This definition is from ISO/TS 15926-7:2011[14] and ISO/TS 15926-8:2011[11].

[SOURCE: ISO 10303-2:2024[6], 3.2.21, modified - Note 1 to entry has been added]

manufactured product class

class (3.2.1) whose members are individuals (3.2.6) produced by a manufacturing process

EXAMPLE 1	Lightbulbs of type “60 W 230 V E27” is a manufactured product class (3.2.9) whose members (3.2.10)are discrete.

EXAMPLE 2	“EN 228” unleaded petrol is a manufactured product class (3.2.9) whose members (3.2.10) are continuous.

Note 1 to entry: The members (3.2.10) of a manufactured product class (3.2.9) can be discrete, or can be batches or continuous flows, such as process fluids.

Note 2 to entry: A manufactured product class (3.2.9) might correspond to a specification that has not been realized, such product specification for which no products have been made.

member

thing (3.1.26) that satisfy classification (3.2.2) criteria for membership

Note 1 to entry: In ISO 15926-2:2003[3], member (3.2.10) and instance (3.2.7) both mean member (3.2.10) of a class (3.2.1), instance (3.2.7) is used when the class (3.2.1) is an entity type in the conceptual data model (3.1.3)), whilst member (3.2.10) of means there is an instance (3.2.7) of the classification (3.2.2) entity type between instance (3.2.7) of the conceptual data model (3.1.3).

power class

set of all subclass (3.2.17) of a class (3.2.1), including the class (3.2.1) itself and the empty set

Note 1 to entry: A subclass (3.2.17) of a power class (3.2.11) selects subclasses (3.2.17) that satisfy a specific context. A restricted set of subclasses (3.2.17) can be a “facet” or “aspect”.

product class

artefact that has been designed by a manufacturer and whose members (3.2.10) may fulfil the requirements set forth in an instance (3.2.7) of an asset requirement class

proprietary class

class (3.2.1) whose specification for membership is owned, controlled, or protected by an organization and is not generally available outside that organization

proprietary product class

class (3.2.1) that is a manufactured product class (3.2.9) and a proprietary class (3.2.13)

EXAMPLE 1	A product specification that is owned by a commercial organization, and that is marketed under and protected by a registered trade name, is the basis for a proprietary product class (3.2.14).

EXAMPLE 2	Lightbulbs of type “60 W 230 V E27” manufactured by Phillips are members (3.2.10) of a proprietary product class (3.2.14)

Note 1 to entry: proprietary product classes (3.2.14) are specializations that depend on rules of inclusion and exclusion some of which are controlled in a closed way. This means that some aspects of the specification can be arbitrarily changed. Many proprietary product classes (3.2.14) are specializations of commodity product classes (3.2.3), de facto classes (3.2.5), or both, where the additional restrictions reflect design or manufacturing details that the manufacturer uses to differentiate its product from others of the same general type.

standard class

class (3.2.1) whose specification for membership is owned or controlled by a standardization body and is publicly available

EXAMPLE 1	ASME B16.9 constrains the dimensions and shapes of steel butt welding pipe fittings.

EXAMPLE 2	IEC 60079–1 specifies constraint (3.13.5) on electrical equipment to ensure standard degrees of explosion-proofness.

Note 1 to entry: standard classes (3.2.15) result from the work of national, international, or industry standardization bodies and cover sizes, shapes, materials, performance, and manufacturing processes of equipment and materials. The rules for exclusion and inclusion (or conformance) are agreed by an open, consensus process and are made publicly available. A standard class (3.2.15) need only constrain one particular aspect and can often be insufficient to determine usage or be a full manufacturing specification.

superclass

class (3.2.1) that is a generalization of one or more other classes (3.2.1)

Note 1 to entry: The classes (3.2.1) being generalized is known as a subclasses (3.2.17).

Note 2 to entry: A particular class (3.2.1) can be a superclass (3.2.16) with respect to one relationship (3.1.23) and simultaneously a subclass (3.2.17) with respect to another relationship (3.1.23).

[SOURCE: ISO/IEC 11179-3:2023[12], 3.1.9, modified — Note 2 to entry has been deleted.]

subclass

class (3.2.1) that is a specialization of another class (3.2.1)

Note 1 to entry: The class (3.2.1) being specialized are known as superclass (3.2.16).

Note 2 to entry: A particular class (3.2.1) can be a subclass (3.2.17) with respect to one relationship (3.1.23) and simultaneously a superclass (3.2.16) with respect to another relationship (3.1.23).

[SOURCE: ISO/IEC 11179-3:2023[12], 3.1.10, modified — Note 2 to entry has been deleted.]

## Terms relating to artefact, product and material object

physical object

<4-Dimensionalism> individual (3.2.6) that is a distribution of energy, matter or both

## Terms relating to part and components

temporal part

state

part of an object that exist in time

temporal part relationship

whole-part relationship (3.4.3) such that the part is all of the whole for a period of time

whole-part relationship

relationship (3.1.23) between two individuals (3.2.6) such that 4D extent of one is part of the 4D extent of the other

## Terms relating to aggregations of parts

engineered item

type of equipment created by specific and unique engineering (3.8.3) and specifications, developed with a supplier or manufacturer

Note 1 to entry: An engineered item (3.5.1) has a unique identifier (UID).

system element

part of a system which can be inanimate physical objects (not alive) and animate physical objects (alive)

Note 1 to entry: A system often is called a “set of elements”.

tagged item

equipment and major electrical and instrumentation item that has a specific tag number and which is treated individually for tracking and tracing purposes

Note 1 to entry: Bulk materials are excluded from this definition which normally are identified batches.

## Terms relating to activity and participation

activity

<4-Dimensionalism> individual (3.2.6) that is something happening or changing

Note 1 to entry: The ISO/TS 23164:2025[1] definition of "activity" is "something that happens or is being done".

Note 2 to entry: The ISO/IEC/IEEE 15288:2023[15]definition of “activity” as “set of cohesive tasks of a process” implies a hierarchy in which processes contain activities, and activities contain tasks. Within this document, all are activities.

Note 3 to entry: The definition in the PMI Lexicon of Project Management Terms states that “activity” is an element of work performed during the course of a project. An activity normally has an expected duration, an expected cost, and expected resource requirements. Activities can be subdivided in tasks.

event

<4-Dimensionalism> individual (3.2.6) that has zero extent in time

managed programme of work

activity (3.6.1) that contains management and planning (3.8.6) for the whole

project

activity (3.6.1) with defined start and finish criteria undertaken to create a product or service in accordance with specified resources (3.6.5) and requirements

Note 1 to entry: A continuing operational activity (3.6.1) is not a project (3.6.4).

Note 2 to entry: A project (3.6.4) has a beginning and end, and can be constrained by available time, funding and resources (3.6.5).

Note 3 to entry: In many cases an activity (3.6.1) that is planned consists of parts of different projects (3.6.4) with different objectives. Therefore the activity (3.6.1) is not a project (3.6.4).

Note 4 to entry: Product here is a result of the project activity (3.6.1), not directly related to Product Breakdown Structure.

[SOURCE: ISO/IEC/IEEE 15288:2023[15], modified — “endeavour” has been changed to “activity”; The original Notes to entry have been deleted.]

resource

individual (3.2.6) that is used to perform an activity (3.6.1)

Note 1 to entry: A resource (3.6.5) can be material or staff time or equipment.

## Terms relating to person, organization, position and role in organization

enterprise

<linguistics> any private or public business or company

maintenance organization

<RDL> organizational unit that is responsible for the maintenance of the reference data library (3.1.22)

Note 1 to entry: Procedure for the maintenance of ISO/TC 184/SC 4 reference data (3.1.20) is described in ISO/TC 184/SC4 N2591.

Note 2 to entry: The maintenance organization represents the combination of the ISO/TC 184/SC 4 secretariat, the ISO/TC 184/SC 4/WG 3 and the ISO/TC 184/SC 4/WG 22 together carrying out the task of maintenance of ISO/TS 15926-4 reference data library published as a database using the procedure. The maintenance team (MT) is defined by the ISO/TC 184/SC 4/WG 3 and the validation team (VT) is defined by the ISO/TC 184/SC 4/WG 22.

stakeholder

person or company that is involved in a particular organization, project (3.6.4) or system

EXAMPLE	Involvement of a person or company can concern safety, environment or other.

Note 1 to entry: The stakeholder's involvement is especially because they have invested money in it or have a functional responsibility.

## Terms relating to requirement, design and specification

asset planning

planning (3.8.6) that is for the construction, commissioning, refurbishment, maintenance, decommissioning, and disposal of assets (3.1.2), including the running down and starting up of production by assets (3.1.2)

designing

activity (3.6.1) of developing by creating, planning, calculation, or laying out for a predetermined purpose

engineering

activity of designing (3.8.2) or producing by methods of technical sciences

Note 1 to entry: During the activity the properties of matter and the sources of energy in nature are made useful for human beings in structures, machines, and products.

functional requirement

requirement defining either the functional capabilities or behaviour, or both, that a product shall have

model based systems engineering

MBSE

formalized application of modelling systems engineering (3.8.10)information (3.1.11)

Note 1 to entry: The application of modelling supports system requirements (3.8.9), design, analysis, verification (3.10.4) and validation (3.10.3) (V&V) activities (3.6.1) with their mutual relationships, beginning in the conceptual design phase and continuing throughout development and later life-cycle phases.

planning

activity (3.6.1) that is creating a plan (3.9.18)

specified requirement

need or expectation that is stated

stakeholder requirement

requirement defining how a stakeholder (3.7.3) wants to interact with an intended solution for a requirement

system requirement

result of the transformation of the stakeholder (3.7.3), user-oriented view of desired capabilities into a technical view of a solution that meets the operational needs of the user

systems engineering

interdisciplinary approach governing the total technical and managerial effort required to transform a set of stakeholder (3.7.3) needs, expectations, and constraints (3.13.5) into a solution

EXAMPLE	A solution can be a system.

Note 1 to entry: The approach supports a solution throughout its life.

technical solution

solution to a problem that is dealt with so that the difficulty is removed by applying an appropriate technology or design principle

## Terms relating to method and plan

baseline schedule

schedule (3.9.22) that is used by a planning activity (3.6.1) as a reference basis for comparison to monitor and control progress on the planned activity (3.6.1)

Note 1 to entry: The PMI Lexicon of project (3.6.4) Management Terms defines baseline schedule (3.9.1) as “the approved version of a schedule (3.9.22) model that can be changed using formal change control procedures and is used as the basis for comparison to actual results”.

Note 2 to entry: A baseline schedule (3.9.1) is a specialization of the ISO/IEC/IEEE 15288:2023[15] definition of “baseline” as “formally approved version of a configuration item, regardless of media, formally designated and fixed at a specific time during the configuration item's life-cycle”.

calendar

pattern of working days and shifts that are available for scheduled activities (3.6.1).

current schedule

schedule (3.9.22) of the planned activity (3.6.1) that is currently specified as the agreed working schedule (3.9.22) by a planning activity (3.6.1) to the performer of the planned activity (3.6.1)

Note 1 to entry: The role of a current schedule (3.9.3) is to avoid adverse schedule (3.9.22) impact.

decision gate

activity (3.6.1) that approves continuation

Note 1 to entry: Continuation of work beyond a decision gate (3.9.4) is contingent on the agreement of the decision-makers.

Note 2 to entry: Criteria for continuation of abandonment are established for each decision gate (3.9.4).

early finish

earliest possible point in time when the uncompleted portions of the activity (3.6.1) can finish based on the schedule (3.9.22)

early start

earliest possible point in time when the uncompleted portions of the activity (3.6.1) can start based on the schedule (3.9.22)

finish to finish

successor relationship (3.1.23) in a plan (3.9.18) that is from the finish of one activity (3.6.1) to the finish of the next

finish to start

successor relationship (3.1.23) in a plan (3.9.18) that is from the finish of one activity (3.6.1) to the start of the next

free float

period of time into which an activity (3.6.1) in a plan (3.9.18) can overrun without causing a delay to subsequent activities (3.6.1) in the plan (3.9.18)

frontline date

date on which the achieved progress on an activity (3.6.1) was scheduled

functional approach

series of three functions that satisfy a need or demand for a demonstration that specified requirements (3.8.7) are fulfilled

Note 1 to entry: The three functions are selection, determination, and review and attestation.

implementation method

technique used by computer systems to exchange or access data (3.1.4)

lag

period in time that is specified for a plan succession link (3.9.19)

late finish

latest point in time that an activity (3.6.1) can finish based on the schedule (3.9.22)

late start

latest point in time that an activity (3.6.1) can start based on the schedule (3.9.22)

live schedule

schedule (3.9.22) that has been revised from the current schedule (3.9.3) to mitigate any delays

milestone

event (3.6.2) that is significant in a project (3.6.4), programme of work, or portfolio

plan

specification of how an activity (3.6.1)will be done

Note 1 to entry: A plan (3.9.18) can include the following: a breakdown into component activities (3.6.1); resources (3.6.5) required by component activities (3.6.1); durations of component activities (3.6.1); required dates for milestones (3.9.17); succession relationships (3.1.23) and lags (3.9.13) between component activities (3.6.1); calendars (3.9.2) for activities (3.6.1) , resources (3.6.5) and lags (3.9.13); start and end dates for component activities (3.6.1).

Note 2 to entry: An unscheduled plan (3.9.18) does not contain start and end dates for component activities (3.6.1). A schedule (3.9.22) does contain start and end dates for component activities (3.6.1).

Note 3 to entry: Plan corresponds to the term “design (noun)” defined in ISO/IEC/IEEE 15288:2023[15], where the design is for an activity (3.6.1) rather than for a system or system element (3.5.2).

Note 4 to entry: A plan (3.9.18) can be more or less detailed.

Note 5 to entry: The PMI Lexicon of Project Management Terms defines “project management plan” as “the document that describes how the project will be executed, monitored and controlled, and closed.”

plan succession link

succession relationship (3.1.23) between one activity (3.6.1) or milestone (3.9.17) and the next in a plan (3.9.18)

revised schedule

schedule (3.9.22) that has been revised from the current schedule (3.9.3)

revision shutdown

shutdown that is in order to make a change to a production activity (3.6.1)

schedule

specification of an activity (3.6.1) that contains all that is required as a basis for execution, and that specifies start and end dates for component activities (3.6.1)

Note 1 to entry: A schedule (3.9.22) is the end product of a planning activity (3.6.1) at a point in time. A schedule (3.9.22) can be revised as a project (3.6.4) progresses.

Note 2 to entry: A schedule (3.9.22) includes calendar (3.9.2), succession relationship (3.1.23), and all other constraint (3.13.5).

start to finish

successor relationship (3.1.23) in which a successor activity (3.6.1) cannot finish until a predecessor activity (3.6.1) has started

start to start

successor relationship (3.1.23) in which a successor activity (3.6.1) cannot start until a predecessor activity (3.6.1) has started

subject of work

physical object (3.3.1) that an activity (3.6.1) is carried out upon

total float

period of time into which an activity (3.6.1) in a plan (3.9.18) can overrun without causing a delay to the completion of the plan (3.9.18) as a whole

Note 1 to entry: total float (3.9.26) is calculated as the latest start time minus the earliest start time, or the latest finish time minus the earliest finish time, whichever is greater.

work order

managed programme of work that contains a request from one party to another for one or more activities (3.6.1) to be performed

## Terms relating to process and service

conformance testing

process to determine whether an implementation meets the requirements of a standard

V-model

graphical representation of a system's development life-cycle

Note 1 to entry: It is used to produce rigorous development life-cycle models and project management models.

validation

proof that the system accomplishes or, toned down, can accomplish its purpose

Note 1 to entry: It is usually much more difficult and much more important to validate a system than to verify it, and give an answer to the question: "Have we made the correct product?"

verification

proof of compliance with the specification

Note 1 to entry: Compliance might be determined by an objective test, analysis, demonstration, inspection, etc. for each requirement or set of requirements. It answers the question: "Have we made the product correctly?"

Note 2 to entry: In general, verification (3.10.4) is seen as the process of checking the compliance with a requirement.

## Terms relating to geometric and topological representation

3D model

representation of a physical body using a collection of interconnected points in a three-dimensional space

Note 1 to entry: Interconnected points can form triangles, lines, curved surfaces.

graph of mapping

set of pairs defined by a mapping, where each consists of a member (3.2.10) of the domain (3.13.11) and a member (3.2.10) of the range

Note 1 to entry: A graph of a mapping is sometimes called an “extension (3.1.9)” of a mapping.

parameterization function

mapping from a numeric space to the principal set (3.11.5), which provides a unique numeric identification for each member (3.2.10) of the principal set (3.11.5)

parameterized

having a parameterization function (3.11.3)

principal set

<geometry and topology> set of things (3.1.26) for which a structure (3.11.6) specifies relationships (3.1.23)

Note 1 to entry: A principal set (3.11.5) of a structure (3.11.6) is sometimes called an “extension (3.1.9)” of a structure (3.11.6).

structure

<geometry and topology> set of things (3.1.26) with a set of relationships (3.1.23)

## Terms relating to physical quantity and physical property

attribute

<data model> data element for the computer-sensible description of a property (3.12.3), a relation or a class (3.2.1)

EXAMPLE	The name of a property (3.12.3), the code of a class (3.2.1), the measure unit in which value (3.13.63)of a property (3.12.3) are provided.

Note 1 to entry: An attribute (3.12.1) describes only one single detail of a property (3.12.3), class (3.2.1) or relation.

Note 2 to entry: In ISO 15926 and other process industry data standards, i.e., CFIHOS[16], attributes (3.12.1) describe details of entities (3.1.7) of the conceptual data model (3.1.3), where property (3.12.3) is one of the entities (3.1.7) .

Note 3 to entry: In RDF[17], which is used by some ISO 15926 parts for implementation, an attribute is a characteristic of an object, and a property is a specific attribute with defined meaning that might be used to describe other resources.

[SOURCE: ISO 13584-42:2010[18], 3.3]

attribute

<linguistics> quality or feature of something

Note 1 to entry: This definition is from ISO/TS 15926-11:2023[8].

property

physical quantity

aspect or quality of something that can be determined by measurement or observation

Note 1 to entry: physical quantity (3.12.3) is added as an admitted term (3.1.25) to align with ISO/IEC Guide 99[19] and ISO 80000-1:2022[20]. A physical quantity (3.12.3) is something observable about phenomenon, body, or substance that has a magnitude that can be expressed as a number and a reference (3.13.33).

Note 2 to entry: A property (3.12.3) is defined by the totality of its associated attributes (3.12.1).

Note 3 to entry: ISO 15926-2:2003[3] entity "property" is an ISO 15926-2:2003[3] entity "class" whose member (3.2.10)individuals (3.2.6) have the same degree or magnitude of a quality or characteristic, and might be quantified by mapping to a number on a scale.

Note 4 to entry: In RDF[17], which is use by some ISO 15926 parts for implementation, an attribute is a characteristic of an object, and a property is a specific attribute with defined meaning that might be used to describe other resources.

## Terms relating to implementation

administrative information

information (3.1.11) about the administration of an item in a reference data library (3.1.22)

EXAMPLE	Creation date, last change date, origin, change description, explanatory comment.

[SOURCE: ISO/IEC 11179-3:2023[21], 3.2.67, modified — The domain has been removed and "registry" has been replaced by "reference data library".]

base template

template (3.13.50) with only entity (3.1.7) types in the expansion of its template axiom (3.13.52)

blank node

BN

node in an RDF graph (3.13.30) representing a resource (3.6.5) for which a literal is not given

Note 1 to entry: The resource represented by a blank node (3.13.3) is also called an anonymous resource. According to the RDF standard a blank node (3.13.3) can only be used as subject or object of an RDF triple (3.13.34).

class template

template (3.13.50) for making statements (3.13.49) about classes (3.2.1)

constraint

<linguistics> thing (3.1.26) that limits something, or limits one’s freedom to do something

EXAMPLE	A design constraint (3.13.5), legal constraint (3.13.5) or implementation constraint (3.13.5).

core RDL

set of RDLs (3.1.22) that only hold core classes (3.2.4) and reference individuals (3.2.6)

Note 1 to entry: Part of the content is normalized.

core template

RDL (3.13.33)template (3.13.50) for which all reference data items (3.1.21) in the expansion of its template axiom (3.13.52) are core classes  (3.2.4)

data store

computer system that allows data (3.1.4) to be stored for future reference (3.13.33)

data type

domain of values (3.13.63)

[SOURCE: ISO 10303-2:2024[6], 3.2.9]

data warehouse

data store (3.13.8) in which related data (3.1.4) are merged to provide an integrated set of data (3.1.4) containing no duplication or redundancy of information (3.1.11), and which supports many different application viewpoints

domain

<simplified industrial usage of reference data> set of all possible independent values (3.13.63) the relationship (3.1.23) can take

Note 1 to entry: It is the collection of all possible inputs (the “left-hand” of a relationship (3.1.23)).

entity data type

representation of an entity (3.1.7)

Note 1 to entry: An entity data type (3.13.12) establishes a domain of values (3.13.63) defined by common attributes (3.12.1) and constraints (3.13.5).

[SOURCE: ISO 10303-2:2024[6], 3.2.10]

entity instance

named unit of data (3.1.4) which represents a unit of information (3.1.11) within the class (3.2.1) defined by an entity (3.1.7)

Note 1 to entry: It is a member (3.2.10) of the domain established by an entity data type (3.13.12).

enumeration

complete, ordered listing of all the items in a collection

Note 1 to entry: The term is commonly used in mathematics and computer science to refer to a listing of all of the elements of a set.

exchange file

computer-interpretable format used for storing, accessing, transferring and archiving data (3.1.4)

first-order logic

symbolized reasoning in which each sentence, or statement (3.13.49), is broken down into a subject and a predicate

Note 1 to entry: The predicate modifies or defines the properties (3.12.3) of the subject. In first-order logic (3.13.16), a predicate can only refer to a single subject.

Note 2 to entry: First-order logic (3.13.16) is also known as first-order predicate calculus or first-order functional calculus.

formal syntax

specification of the valid sentences of a formal language using a formal grammar

EXAMPLE	An Extensible Markup Language (XML) document type definition (DTD) is a formal syntax (3.13.17).

Note 1 to entry: A formal language is computer-interpretable.

[SOURCE: ISO 8000-2:2022[22], 3.9.1, modified — EXAMPLE 2 and 3, Note 2 and 3 to entry have been deleted.]

individual template

template (3.13.50) for making statements (3.13.49) about individuals (3.2.6)

information exchange layer

particular conceptual model based on the software implementation layers (3.13.46) that characterizes the information (3.1.11) exchange between computing system

ISO 15926-2 language

first-order language in which the ISO 15926-2:2003[3] data model is expressed

Manchester syntax

user-friendly compact syntax for OWL 2 ontologies which is frame-based, as opposed to the axiom-based syntaxes for OWL 2

N-triple

line-based, plain text format for encoding an RDF graph (3.13.30)

object information model

OIM

ontology (3.1.18) of classes (3.2.1) and relations for which a particular class (3.2.1) is singled out for characterization

Note 1 to entry: In Description Logic terminology, it is a TBox.

OWL native

modelling style in which a relationship (3.1.23) is expressed as an RDF predicate

payload data

actual data (3.1.4) in a data (3.1.4) packet or data (3.1.4) container minus all headers attached for transport and minus all descriptive meta-data

Note 1 to entry: In a network packet, headers are appended to the payload for transport and then discarded at their destination.

punning

<information technology> declaring a class (3.2.1) and an individual (3.2.6), having the exact same identifier, in order to use them in different model (3.1.16) constructions, which would be illegal to use if applied to one object

Note 1 to entry: Punning can also be applied to two property (3.12.3) declarations of different property (3.12.3) type.

punning

<OWL Direct Semantics> assigning the same name to objects that are treated as different in OWL Direct Semantics

range of relationship

<simplified industrial usage of reference data > set of all possible dependent values (3.13.63) the relationship (3.1.23) can produce from the domain (3.13.11) values

Note 1 to entry: The domain (3.13.11) values are in the “right-hand” of a relationship (3.1.23).

RDF/XML

format with an XML syntax for RDF, as defined in the W3C recommendation "RDF/XML Syntax Specification (Revised)"

RDF graph

resource description framework graph

graph structure formed by a set of RDF triples (3.13.33)

[SOURCE: RDF 1.1 Concepts and Abstract Syntax[23]]

RDF schema

resource description framework schema

language for describing vocabularies in RDF which is a semantic extension (3.1.9) of RDF, providing mechanisms for describing groups of related resources and the relationships (3.1.23) between these resources

RDF statement

resource description framework statement

statement (3.13.49) stating facts, relationships (3.1.23) and data (3.1.4) by linking resources of different kinds

RDL template

reference data library template

template (3.13.50) with at least one reference data item (3.1.21) in the expansion of its template axiom (3.13.52)

RDF triple

resource description framework triple

atomic data entity in the Resource Description Framework (RDF) data model

Note 1 to entry: An RDF-triple (3.13.34) represents a relationship between the objects or data (3.1.4) that it links.

Note 2 to entry: A triple comprises at least:

— an object called “subject”;

— a predicate (also called property) that denotes a relationship (3.1.23) between a subject and an object;

— an object or data called “object”.

[SOURCE: RDF 1.1 Concepts and Abstract Syntax[23]]

reference data item definition by document reference

normative reference to a document (3.1.5) that provides the normative definition of a reference data item (3.1.21)

reference data item definition by text

text, and optionally equations and figures that are intended to be understood by a human, and that is the normative definition of a reference data item (3.1.21)

reference data item non-human-interpretable identifier

text that is a unique identifier for a reference data item (3.1.21) and that is not intended to be interpreted by a human

reference data item human-interpretable identifier

text that is a unique identifier for a reference data item (3.1.21), and that is intended to be interpreted by a human

Note 1 to entry: If a reference data item (3.1.21) is a class (3.2.1), then its reference data item human-interpretable identifier (3.13.38) can be a term (3.1.25).

Note 2 to entry: The reference data item human-interpretable identifier (3.13.38) is used for all references to a reference data item (3.1.21) in natural language text definitions.

Note 3 to entry: Names in natural language and terms (3.1.25) in dictionaries are often unique only within a context. A reference data item (3.1.21) can be used in many contexts. Hence a reference data item human-interpretable identifier (3.13.38) can be longer than a name in a natural language, and contain words which qualify a natural language name, to ensure that it is unique.

Note 4 to entry: A reference data library (3.1.22) can contain additional terms (3.1.25) for a reference data item (3.1.21), which are not unique and which are used within particular contexts.

reference data library module

collection of reference data items (3.1.21) within a reference data library (3.1.22) that shares the same subject area

EXAMPLE	Electrical is a reference data library module (3.13.39), represented by a specific spreadsheet, which contains reference data library items (classes (3.2.1)) that are of direct interest to the electrical engineering discipline, which in this case is the subject area. In the context of ISO 15926-2:2003[3], the reference data library module (3.13.39) is a class of class with as members all classes contained by the corresponding spreadsheet.

reification

modelling style in which a relationship (3.1.23) is expressed as an object class (3.2.1)

EXAMPLE	The relation Employed-by is reified by the object Employment which is connected to the objects Employee and Organization. The meaning of the relation with cardinalities at both ends is “an organization has zero or more employees”. The reified Employment object can be subject in other relations, defining it.

Note 1 to entry: The relational entity data types of ISO 15926 are all the entity data types which have exactly two attributes (3.12.1), except ISO 15926-2 entity “class of relationship”.

semantic data modelling

development of descriptions and representations of data (3.1.4) in such a way that the latter’s meaning is explicit, accurate, and commonly understood by both humans and computer systems

Note 1 to entry: In semantic data modelling (3.13.41), all concepts used to model a system are explicitly defined by ontologies (3.1.17), capturing the “meaning” of data (3.1.4) with all its inherent relationships (3.1.23) in a single graph.

semantic encoding

concept encoding

technique of replacing natural language terms (3.1.25) in a message with identifiers that reference data dictionary entries

EXAMPLE	ISO 8000-110:2021[24] specifies how semantic encoding (3.13.42) supports the exchange of master data that is characteristic data.

Note 1 to entry: By applying semantic encoding (3.13.42) to data (3.1.4), an organization creates a basis for portable data (3.1.4) by ensuring the semantics (3.13.43) of the data (3.1.4) are explicit.

Note 2 to entry: Semantic encoding (3.13.42) is necessary to create characteristic data, where the replaced natural language terms (3.1.25) are properties (3.12.3) (for each of which the data set includes a corresponding value (3.13.63)).

[SOURCE: ISO 8000-2:2022[22], 3.9.2]

semantics

study of meaning, concerned with the relationship (3.1.23) between signifiers that people use when interacting with the world, and the things (3.1.26) in that world that these signifiers denote

EXAMPLE 1	The signifiers can be words, phrases, signs, and symbols.

EXAMPLE 2	Things (3.1.26) denoted by signifiers can be entities (3.1.7), concepts, ideas.

Note 1 to entry: The goal of semantics (3.13.43) is the creation of a common understanding of the meaning of things (3.1.26), helping people understand each other despite different experiences or points of view.

signature

named, ordered and typed list of template roles (3.13.57)

signature statement

statement (3.13.49) that states that the performer of an activity (3.6.1) applied including the sign-off date on an individual Named Graph

software implementation layer

conceptual model that characterizes the software implementation

SPARQL endpoint

conformant to SPARQL protocol service as defined in W3C's SPARQL Protocol for RDF (SPROT)

specialized template

set of statements (3.13.49) about individuals (3.2.6) or classes (3.2.1), that is subclass (3.2.17) of a core template (3.13.7) or another specialized template (3.13.48), and which has one or more restrictions on its roles

statement

information (3.1.11) that is regarded as indivisible and which is the case, independent of natural language

Note 1 to entry: A statement (3.13.49) can be recorded as an instance (3.2.8) of the entity relationship in ISO 15926-2:2003[3]. A set of one or more statements (3.13.49) can be recorded in shorthand form as a single item as an instance (3.2.8) of a template (3.13.50), as defined in ISO/TS 15926-7:2011[14].

template

<ISO 15926 implementation method with template methodology> set comprising of a first-order logic (3.13.16) predicate for which a definition is stated as an axiom, a template signature (3.13.58) and a template axiom (3.13.52) expansion

template

<ISO 15926 implementation method with template methodology and OWL> n-ary predicate, represented in OWL reified form as a class (3.2.1) with one functional property (3.12.3) (role) per variable

template axiom

axiom in the template language (3.13.56) defining the interpretation of template statements (3.13.59)

template expansion

template axiom expansion

statement (3.13.49) expressed in ISO 15926-2:2003[3] entity data types, equivalent to a template axiom (3.13.52)

Note 1 to entry: The expansion of a template axiom (3.13.52) refers to a typically complex, biconditional in the ISO 15926-2 language (3.13.20). It is obtained by repeated application of template biconditionals until the template’s interpretation is expressed directly in terms of simple ISO 15926-2 language (3.13.20) constructions.

template instance

ordered list of entity instances (3.13.13) of which a template (3.13.50) is true

Note 1 to entry: In OWL, the template instance (3.13.54) is an individual (3.2.6) with role relationships (3.1.23) to the individuals (3.2.6) that instantiate the template (3.13.50). The atomic/ground statement (3.13.49) is made by replacing the variables of the template (3.13.50) with OWL individuals (3.2.6).

template instance expansion

set of simple statements (3.13.49) in the ISO 15926-2 language (3.13.20), obtained by instantiating variables in the expanded template axiom (3.13.52) with entity instances (3.13.13)

template language

axioms in first-order logic (3.13.16) extending the ISO 15926-2:2003[3] data model

template role

named and numbered argument in a template with required type given as entity type, data type (3.13.9), or reference data class

EXAMPLE	Template InstanceOfIndirectProperty(a, b, c) means that a is a ClassOfIndirectProperty, b a (temporal part of) PossibleIndividual to which the relation applies and c the instance of Property. b has an a type of ClassOfIndirectProperty, which has c instance of Property. It has the following roles:

a) role name: Property type;

b) role name: Property possessor;

c) role name: Property.

template signature

named, ordered and typed list of template roles (3.13.57)

EXAMPLE	Template InstanceOfIndirectProperty(a, b, c) means that a is a ClassOfIndirectProperty, b a (temporal part of) PossibleIndividual to which the relation applies and c the instance of Property. b has a type of ClassOfIndirectProperty, which has a c instance of Property. The template signature (3.13.58) is:

a) role name: Property type, role type: ClassOfIndirectProperty;

b) role name: Property possessor, role type: PossibleIndividual;

c) role name: Property, role type: Property.

template statement

statement (3.13.49) made by instantiating the roles of a template (3.13.50) with entity instances (3.13.13)

triple

representation of a relation between the objects or data that it links

Note 1 to entry: A triple comprises at least:

— an object called “subject”;

— a predicate (also called property) that denotes a relationship (3.1.23) between a subject and an object;

— an object or data called “object”.

triple store

data store (3.13.8) capable of storing triples (3.13.33)

upper camel-case notation

naming convention in which compound words are joined together without spaces, and the first letter of every word is uppercase

[SOURCE: ISO/IEC 39794-5:2019[25], 3.56]

value

unit of data (3.1.4)

[SOURCE: ISO 10303-2:2024[6], 3.2.33]

(informative)

Development Approach

Introduction

The annex describes the development approach of the following:

— updating terms with new or revised ISO 15926 parts.

— terms that are also used as data model entities or RDIs.

— terms that have different definitions in different ISO 15926 parts.

Approach for updating terms with new or revised ISO 15926 parts

This document consolidates terms from eleven (11) published ISO 15926 parts into a single vocabulary standard, which eliminates duplication of terms, reduces the time and effort required to revise existing parts and to develop new parts of the ISO 15926 series, and enables a unified vocabulary to be shared with external stakeholders, such as participants from the process industry, academia, and other standards development organizations.

The definitions of terms in this document are taken from published ISO 15926 parts, without any modification.

The definitions of terms of individual ISO 15926 parts are continuously updated when necessary.

EXAMPLE	The term statement is defined in ISO/TS 15926-11:2015[26]as “information that is regarded as indivisible”, and in ISO/TS 15926-11:2023[8], the definition is updated as “information that is regarded as indivisible and which is the case, independent of natural language”.

The project team responsible for this document will coordinate with teams developing new or revised ISO 15926 parts. Definitions of terms will be updated accordingly if necessary, and revised editions will be released on a regular basis.

Future editions of this document will also include alignments with ISO/TS 23164:2025[1], which specifies a vocabulary for industrial data that defines generic terms for things that exist in more than one industrial domain, to improve terminological consistency.

Approach to terms that are used as data model entities or RDIs

Background

Vocabulary scope

This document contains all terms published in the ISO 15926 series. It aims to eliminate duplication of terms, reduce the effort and time required for revising existing parts and developing new parts.

The scope of this document states that the entities used in the ISO 15926 data model and RDIs in the ISO/TS 15926-4 RDL are outside of the scope. However, certain words or phrases, such as class, individual and property, are used both as defined terms and as data model entities or RDIs.

Entities and RDIs of ISO 15926

The ISO 15926 series is uniquely positioned to address the process plant needs for integrating life-cycle data. It consists of multiple parts, including data models, reference data and implementation methods.

The architecture that underlies ISO 15926 is illustrated in Figure A.1. It supports integration over a wide context due to the nature of the generic, conceptual data model that is its foundation. Process plant life-cycle data is structured according to the data model defined in ISO 15926-2:2003[3], which contains 201 entities. This data is divided into data about an individual process plant, and reference data, which represents information that is common to many process plants or of interest to many users.

Figure A.1 — ISO 15926 data model and reference data

Relationship between Vocabulary with Entities and RDIs

ISO/IEC 5394:2024[27]describes that the level of structure associated with a given concept system corresponds with the strength and richness of the concept relations among concepts. Concept systems can be classified by the complexity of their structure, scaled starting with: weakly structured concept systems, hierarchical concept systems, hybrid concept systems and reason-based concept systems with machine-readable semantics, as shown in Figure A.2.

Weakly structured concept system, such as Glossary: a concept system in which concepts are collected as simple sets of concepts or the concept relation is not obvious, and the terms assigned to concepts are mostly displayed in a simple way or alphabetical order.

Hierarchical concept system, such as Taxonomy: a concept system in which all the concepts relate to each other hierarchically, using generic relation, partitive relation and so on.

Hybrid concept system, such as Terminology: a concept system constructed using a combination of concept relations, including hierarchical relations and associative relations.

Reason-based concept system, such as Ontology: a concept system in which all the concepts have precise mathematical descriptions, the concepts and their concept relations are clearly and precisely defined. The concept system can be used for reasoning, usually dependent on some sort of semantic representation such as OWL[7] and RDF[17].

Figure A.2 — Level of structure of concept systems

Glossaries and ontologies represent opposite ends of conceptual system complexity, from weakly structured to highly structured systems:

A glossary is a simple, weakly structured system consisting of a list of terms and definitions with minimal or no formal relationships between concepts. It primarily supports basic reference and consistency in terminology by providing definitions for human use, often organized alphabetically.

In contrast, an ontology is a highly structured, formal concept system with precise, machine-readable semantics. Ontologies define not only terms but also the relationships, properties, and axioms that connect them.

While glossaries focus on human-readable definitions for communication, ontologies expand this by capturing the deeper, formal relationships between concepts, supporting both human understanding and machine processing.

This document provides a glossary for process plants, and ISO 15926 data model entities and RDIs provide an ontology for process plants.

Development Approach

A recommendation is made on the use of a term in a vocabulary to refer to an entity: A term in a vocabulary should be used to refer to an entity, only if the entity is a reasonable interpretation of the concept. There can be more than one entity that is a reasonable interpretation of a concept. In a single namespace, a term can be used for only one entity. A term can be qualified to refer to a different entity or can be in a different namespace.

EXAMPLE 1	All 201 entities of ISO 15926-2:2003[3] belong to the concept of term class, including ISO 15926-2:2003[3] entity “class” itself. Therefore, to avoid ambiguity and confusion between the term class and the entity “class”, the term class should not refer to the entity “class”.

In the 11 published parts of ISO 15926, 9 terms are used as ISO 15926-2:2003[3] entities:

activity

class

classification

event

individual

physical object

property

relationship

thing

Among these 9 terms, the definitions of the following 6 terms refer to the definition of an entity in ISO 15926-2:2003[3]:

activity

classification

event

individual

physical object

thing

Since those definitions are reasonable interpretations of the corresponding concepts, the definitions of those terms remain unchanged.

EXAMPLE 2	The term thing, as defined in ISO 15926-6:2024[28], adopts the definition from ISO 15926-2:2003[3]: “anything that is, or can be thought about, or perceived”. The definition is a reasonable interpretation of the “thing” concept.

The definitions of the following 3 terms do not refer to the definition of an entity in ISO 15926-2:2003[3]:

class

property

relationship

The development approach for these 3 terms is adding Notes of entry to explain the specific information relating to the use of the term as the ISO 15926-2:2003[3] entity.

EXAMPLE 3	The term relationship, as defined by ISO 15926-6:2024[28], adopts the definition from ISO/IEC 11179-3:2023[29]: “connection among model elements”. A Note to entry has been added to explain the specific information relating to the use of the term as ISO 15926-2:2003[3] entity: “ISO 15926-2:2003[3] entity “relationship” is something that one thing has to do with another. ISO 15926-2:2003[3] uses the spatio-temporal paradigm. ISO 15926-2:2003[3] entity “relationship” does not exist in space-time”.

Approach to terms that have different definitions in different ISO 15926 parts

In the 11 published parts of ISO 15926, 6 terms have different definitions in different parts:

attribute

entity

instance

interoperability

ontology

punning

Since each term is used to define more than one concept, separate terminological entries are created for each concept, a domain is added in angle brackets before the corresponding definition if necessary.

EXAMPLE	The term punning is defined in ISO/TS 15926-8:2011[11] as “declaring a class and an individual, having the exact same identifier, in order to use them in different model constructions, which would be illegal to use if applied to one object”, and in ISO/TS 15926-12:2018[30] as “assigning the same name to objects that are treated as different in OWL Direct Semantics”. Two terminological entries for punning are created. The domain <information technology> is added before the definition of "punning" from ISO/TS 15926-8:2011[11], and <OWL Direct Semantics> is added before the definition of “punning” from ISO/TS 15926-12:2018[30].

Bibliography

[1]	ISO/TS 23164:2025, Automation systems and integration — Core vocabulary for industrial data

[2]	ISO/TS 15926-4:2024, Industrial automation systems and integration — Integration of life-cycle data for process plants including oil and gas production facilities — Part 4: Core reference data

[3]	ISO 15926-2:2003, Industrial automation systems and integration — Integration of life-cycle data for process plants including oil and gas production facilities — Part 2: Data model

[4]	ISO 55000:2024, Asset management — Vocabulary, overview and principles

[5]	ISO TR 9007:1987, Information processing systems - Concepts and terminology for the conceptual schema and the information base

[6]	ISO 10303-2:2024, Industrial automation systems and integration — Product data representation and exchange — Part 2: Vocabulary

[7]	OWL Web Ontology Language Guide, W3C Recommendation, 10 February 2004

[8]	ISO/TS 15926-11:2023, Industrial automation systems and integration — Integration of life-cycle data for process plants including oil and gas production facilities — Part 11: Simplified industrial usage of reference data based on RDFS methodology

[9]	ISO 22745-2:2010, Industrial automation systems and integration — Open technical dictionaries and their application to master data — Part 2: Vocabulary

[10]	ISO/IEC 11179-1:2023, Information technology — Metadata registries (MDR) — Part 1: Framework

[11]	ISO/TS 15926-8:2011, Industrial automation systems and integration — Integration of life-cycle data for process plants including oil and gas production facilities — Part 8: Implementation methods for the integration of distributed systems: Web Ontology Language (OWL) implementation

[12]	ISO/IEC 11179-3:2023, Information technology — Metadata registries (MDR) — Part 3: Metamodel for registry common facilities

[13]	ISO 1087:2019, Terminology work and terminology science — Vocabulary

[14]	ISO/TS 15926-7:2011, Industrial automation systems and integration — Integration of life-cycle data for process plants including oil and gas production facilities — Part 7: Implementation methods for the integration of distributed systems: Template methodology

[15]	ISO/IEC/IEEE 15288:2023, Systems and software engineering — System life cycle processes

[16]	CFIHOS, Capital Facility Information Handover Specification (IOGP JIP-36)

[17]	Resource Description Framework (RDF) Model and Syntax Specification (1999-02-22)

[18]	ISO 13584-42:2010, Industrial automation systems and integration — Parts library — Part 42: Description methodology: Methodology for structuring parts families

[19]	ISO/IEC Guide 99, International vocabulary of metrology — Basic and general concepts and associated  terms (VIM)

[20]	ISO 80000-1:2022, Quantities and units — Part 1: General

[21]	ISO/IEC 11179-3:2023, Information technology — Metadata registries (MDR) — Part 3: Metamodel for registry common facilities

[22]	ISO 8000-2:2022, Data quality — Part 2: Vocabulary

[23]	RDF 1.1 Concepts and Abstract Syntax, W3C Recommendation 25 February 2014

[24]	ISO 8000-110:2021, Data quality — Part 110: Master data: Exchange of characteristic data: Syntax, semantic encoding, and conformance to data specification

[25]	ISO/IEC 39794-5:2019, Information technology — Extensible biometric data interchange formats — Part 5: Face image data

[26]	ISO/TS 15926-11:2015, Industrial automation systems and integration — Integration of life-cycle data for process plants including oil and gas production facilities — Part 11: Methodology for simplified industrial usage of reference data

[27]	ISO/IEC 5394:2024, Information technology — Criteria for concept systems

[28]	ISO 15926-6:2024, Industrial automation systems and integration — Integration of life-cycle data for process plants including oil and gas production facilities — Part 6: Rules for the development and validation of reference data of ISO/TS 15926-4

[29]	ISO/IEC 11179-3:2023, Information technology — Metadata registries (MDR) — Part 3: Metamodel for registry common facilities

[30]	ISO/TS 15926-12:2018, Industrial automation systems and integration — Integration of life-cycle data for process plants including oil and gas production facilities — Part 12: Life-cycle integration ontology represented in Web Ontology Language (OWL)